c

c *** This code is applied to the S11 model of the 2009 Benchmark test

c     (This code is the same as omega2009-v2a.f)

c

c   * Parameters :

c     MOM : Number of Omega

c     MOB : NUmber of Observation Points

c     MFL : Number of Sub-Faults

c

メインプログラム

サブルーチン

・データ入力

・地震動計算

・結果出力

PROGRAM omega2x

C

IMPLICIT REAL*8(A-H,O-Z)

PARAMETER(MOM=8211,MOB=51,MFL=203)

COMPLEX*16 GX(MOM,MOB),GY(MOM,MOB),GZ(MOM,MOB),Frate(MOM,MFL,2)

DIMENSION rate(mom,MFL,2)

C

WRITE(*,*)

WRITE(*,*)'"omega2009.f" simulates broad-band ground motions'

WRITE(*,*)'   using the Boore source model and the Green function'

write(*,*)'   of the direct S-wave'

write(*,*)' (Originally made in 1992, and modified in 2009)'

WRITE(*,*)

*' For questions, e-mail to "hisada@cc.kogakuin.ac.jp".'

WRITE(*,*)' This code is open for the academic use only.'

C

CALL INDAT(MOM,MOB,MFL,Frate,rate)

CALL HKQVW(MOM,MOB,MFL,Frate,GX,GY,GZ)

CALL OTDAT(MOM,MOB,GX,GY,GZ)

WRITE(*,*)

WRITE(*,*)'Use FFT program "grfftpxA.f" to plot waveforms'

C

STOP

END

C

c

サブルーチン

・データ入力、

震源データ作成

C *** DATA INPUT ***

C

SUBROUTINE INDAT(MOM,MOB,MFL,Frate,rate)

C

IMPLICIT REAL*8(A-H,O-Z)

COMPLEX*16 Frate(MOM,MFL,2),cpx,Ffunc

DIMENSION rate(mom,MFL,2),sdrop(203)

CHARACTER INFILE*40

COMMON /FREQ/DOM,IOM,NOM /OBS/XO(51),YO(51),ZO(51),NOB

*    /time/dt,period  /ntime/nt  /STRC/DNS,VP,QP,FP,VS,QS,FS

*    /FLT/STR,DIP,RAK(203),FL,FW,FD(203),VR,TLc,Pn(203)

*        ,XF(203),YF(203),TD(203),DP(203),fc(203),fmax(203)

*    /Hypo/xhypo,yhypo,zhypo /Radiation/fR1,fR2,fran

*    /IFLT/NF,NSTR,NDIP,NCF,NFfunc /SIGN/ISIGN /Nboore/Niter,idum

c

c *** Check of Dimensions of Common Paramters ***

c

nc=51

if(nc.lt.MOB)then

write(10,*)'from Indat:'

WRITE(10,*)' NC < MOB !!!!!!!!!!!!!'

write(*,*)'from Indat:'

WRITE(*,*)' NC < MOB !!!!!!!!!!!!!'

STOP

END IF

c

nc=203

if(nc.lt.MFL)then

write(10,*)'from Indat:'

WRITE(10,*)' NC < MFL !!!!!!!!!!!!!'

write(*,*)'from Indat:'

WRITE(*,*)' NC < MFL !!!!!!!!!!!!!'

STOP

END IF

C

C *** OMEGA DATA ***

C

write(*,*)

WRITE(*,*)'Enter File name for omega2x-in.csv'

read(*,'(a20)')infile

OPEN (UNIT=7,FILE=infile,STATUS='OLD')

c      OPEN (UNIT=7,FILE='omega2009-ex1.csv',STATUS='OLD')

C

pi2=datan(1.d0)*8.d0

read(7,*)

read(7,*)

read(7,*)dt,nt

write(*,*)

write(*,*)'delta time (s)and number of times =',dt,nt

read(7,*)

read(7,*)period

write(*,*)'minimum period (s) =',period

duration=dt*nt

dom=pi2/duration

iom=1

nom=int(duration/period)

WRITE(10,*)'FROM Indat:'

WRITE(10,*)

WRITE(10,1130)nt,dt,period

WRITE(10,1140)NOM,DOM

1130 format(/' Num. of Time, Delta Time and Min. Period (s)=',i5,2f8.4)

1140 format( ' Num. of Omega, and Delta Omega (rad/sec) =',i5,f10.7)

C

IF((NOM*2).GT.MOM)THEN

write(10,*)'from Indat:'

WRITE(10,*)' NOM*2 >M OM !!!!!!!!!!!!!'

write(*,*)'from Indat:'

WRITE(*,*)' NOM*2 > MOM !!!!!!!!!!!!!'

STOP

END IF

C

C *** MEDIUM DATA ***

C

read(7,*)

read(7,*)

READ(7,*)DNS,VP,QP,FP,VS,QS,FS

C

WRITE(*,2200)

WRITE(10,2200)

WRITE(*,2210)DNS,VP,QP,FP,VS,QS,FS

WRITE(10,2210)DNS,VP,QP,FP,VS,QS,FS

2200 FORMAT(//' Density,Vp(real),   Qp  ,  Fp  ,Vs(real),   Qs  ,  Fs')

2210 FORMAT(F9.3,2(F9.2,F9.3,F7.3))

C

C *** FAULT DATA ***

C

read(7,*)

read(7,*)

READ(7,*)SLENGTH,DWIDTH,NSTR,NDIP

write(*,*)

write(*,*)'Fault Model Parameters:'

write(*,*)'  Fault L and W (m) =',SLENGTH,DWIDTH

write(*,*)'  Numbers of sub-fault along L and W =',NSTR,NDIP

NF=NSTR*NDIP

sNF=dsqrt(dfloat(NF))

IF(NF.GT.MFL)THEN

write(10,*)'from Indat:'

WRITE(10,*)' NF.GT.MFL !!!!!!'

write(*,*)'from Indat:'

WRITE(*,*)' NF.GT.MFL !!!!!!'

STOP

END IF

c

fl=slength/nstr

fw=dwidth/ndip

read(7,*)

c      READ(7,*)DELAY,STR,DIP,VR,TTd,irs

c      Tfc=1.d0/TTd

READ(7,*)DELAY,STR,DIP,VR,dtime,idum

write(*,*)'  Starting Time (s) and Vr (m/s)=',DELAY,Vr

write(*,*)'  Strike and Dip Angles (deg) =',STR,DIP

c      write(*,*)'  Total Faulting Duration (s) and fc (Hz) =',TTd,Tfc

write(*,*)'  Average Random Delay Time (s) =',dtime

str=str*pi2/360.d0

dip=dip*pi2 /360.d0

read(7,*)

READ(7,*)XHYPO,YHYPO,ZHYPO

write(*,*)'  Hypocenter Location (X,Y,Z: m)   =',XHYPO,YHYPO,ZHYPO

read(7,*)

READ(7,*)xflt,yflt,zflt

write(*,*)'  Fault Origin Location (X,Y,Z: m) =',xflt,yflt,zflt

read(7,*)

c

FDsum=0.d0

do 150 j=1,NF

READ(7,*)i,FD(j),rak(j),sdrop(j),fc(j),fmax(j),Pn(j)

rak(j)=rak(j)*pi2/360.d0

FDsum=FDsum+FD(j)

150 continue

FDavg=FDsum/dfloat(NF)

SM=DNS*VS*VS

SM0=SM*FDavg*SLENGTH*DWIDTH*1.0D+10

SM0avg=SM0/dfloat(NF)

write(*,*)

write(*,*)'  Average Slip (cm)     =',FDavg*100.d0

write(*,*)'  Seismic Moment(dyne-cm)=',SM0

sMw=(dlog10(SM0)-16.1d0)/1.5d0

sMj=(dlog10(SM0)-17.72d0)/1.17d0

write(*,*)'  Mw (by Hanks and Kanamori,1979) =',sMw

write(*,*)'  Mj (by M.Takemura, 1990)        =',sMj

c

write(*,*)

sfac=1.d0/3.d0

sSM0=SM0avg/sNF

do 170 j=1,NF

if(sdrop(j).eq.0.d0)goto 170

fc(j)=(4.9d+6)*(sdrop(j)/sSM0)**sfac*Vs/1000.d0

170 continue

c

c *** Sub-fault parameters ***

c

c   * Dip direction *

dcs=fw*dcos(dip)

cst=dcos(str)

sst=dsin(str)

c

iflt=0

ihyp=1

dismin=999999999999999.d0

do 200 i=1,ndip

dc=dcs*(dfloat(i-1)+0.5d0)

zc=zflt-dc*dtan(dip)

if(zc.lt.0.d0)then

write(*,*)'Fault Depth is less than 0!: Dip Number=',i

c          stop

end if

c     * Strike direction *

do 220 j=1,nstr

iflt=iflt+1

sc=fL*(dfloat(j-1)+0.5d0)

dp(iflt)=zc

zz=zc-zhypo

xf(iflt)=xflt+sc*cst+dc*sst

yf(iflt)=yflt+sc*sst-dc*cst

xx=xf(iflt)-xhypo

yy=yf(iflt)-yhypo

dis=dsqrt(xx*xx+yy*yy+zz*zz)

if(dis.lt.dismin)then

ihyp=iflt

・破壊開始時間

（点震源では使用せず）

dismin=dis

end if

td(iflt)=delay+dis/VR

220   continue

200 continue

c

do iflt=1,NF

・ランダム破壊開始時間

（点震源では使用せず）

if(iflt.ne.ihyp)then

td(iflt)=td(iflt)+dtime*dabs(gasdev(idum))

end if

end do

C

WRITE(10,2300)NF

WRITE(10,2305)STR,DIP

WRITE(10,2315)slength,dwidth

WRITE(10,2320)FL,FW

WRITE(10,2330)FDavg,SM0

WRITE(10,2335)sMw,sMj

2300 FORMAT(//'Seismic Source Data:'

*      ,/'  Total Number of Sources =',I3)

2305 FORMAT( '  Strike Angle (rd)=',F12.8,',  Dip Angle   (rd)=',F12.8)

2315 FORMAT( '  Fault Length  (m)=',F12.3,',  Width(m)        =',F12.3)

2320 FORMAT( '  Sub-Length    (m)=',F12.3,',  Sub-Width(m)    =',F12.3)

2330 FORMAT( '  Avgerage Slip (m)=',F12.3,

*        ',  Seismic Moment (dyne-cm)=',e12.4)

2335 FORMAT( '  Moment Magnitude =',F12.1,',  JMA Magnitude   =',F12.1)

c

II=0

DO 270 I=1,NDIP

dc=dcs*(dfloat(i-1)+0.5d0)

zc=zflt-dc*dtan(dip)

DO 270 J=1,NSTR

II=II+1

sc=fL*(dfloat(j-1)+0.5d0)

zz=zc-zhypo

xf(ii)=xflt+sc*cst+dc*sst

yf(ii)=yflt+sc*sst-dc*cst

xx=xf(ii)-xhypo

yy=yf(ii)-yhypo

dis=dsqrt(xx*xx+yy*yy+zz*zz)

WRITE(10,2350)II,I,J

WRITE(10,2360)FD(ii),rak(ii)

write(10,2370)XF(II),YF(II),DP(II)

write(10,2375)sdrop(ii),fc(ii),fmax(ii)

write(10,2380)TD(II),dis/VR,TD(II)-dis/VR

270 CONTINUE

2350 FORMAT(/'   Source Number   =',I4,' (',I2,',',I2,')')

2360 FORMAT( '   Dislocation (m)=',F12.3,', Rake Ang. (rad)=',F9.5)

2370 FORMAT( '   Source X, Y, and Z Location (m) =',3f9.1)

2375 format( '   Stress Drop (bar), fc & fmax (Hz) =',f8.2,2f9.3)

2380 FORMAT( '   Total Time Delay, r/Vr & delta Tr (s) =',3f9.3)

c

c *** Input and compute Boore's Source Parameters ***

c

・Boore型震源モデル

・理論・等方放射係数の接続振動数帯域

call Boore(MOM,MFL,Frate,rate)

C

C *** Radiation Pattern for S and P Waves at high Frequencies ***

C

read(7,*)

read(7,*)

READ(7,*)fR1,fR2

c      write(*,*)fR1,fR2

write(10,2410)fR1,fR2

2410 FORMAT(/' Transient Corners Frequencies (Hz) = ',2E12.4)

C

C *** Selection of F-function, which adjusts the difference of the slip

c     functions between the whole- and sub-faults ***

C

read(7,*)

read(7,*)

READ(7,*)NFfunc,TLc

c      write(*,*)NFfunc,TLc

WRITE(10,2500)NFfunc,TLc

2500 FORMAT(/' Choice of F-function and TLc (s)',i5,f8.2)

c

c *** Output of F-function ***

c

write(*,*)'  F-function (No.1) is stored in F-func.csv'

open(21,file='F-func.csv')

write(21,*)'freq.(Hz),',' F-func'

CPX=(0.D0,1.D0)

pai=4.d0*datan(1.d0)

do io=iom,nom

OM=DOM*DFLOAT(IO)

freq=om/(2.d0*pai)

OMTd=OM*TLc

c   *  Irikura's F-function (1987) *

if(NFfunc.eq.0)then

sinc=dsin(OMTd)/OMTd

Ffunc=1.d0+(sNF-1.d0)*sinc*exp(cpx*OMTd)

c   *  Exp-type Slip Velocity Function *

else

Ffunc=1.d0+(sNF-1.d0)/(1.d0-cpx*OMTd)

end if

write(21,2250)freq,cdabs(Ffunc)

end do

close(21)

2250 format(f12.4,',',e12.4)

C

C *** Selection of Green's Function of Homogeneous Full Space ***

C

read(7,*)

read(7,*)

READ(7,*)NCF

c      write(*,*)NCF

write(10,2600)

IF(NCF.EQ.0)WRITE(10,2610)

IF(NCF.EQ.1)WRITE(10,2620)

IF(NCF.EQ.2)WRITE(10,2630)

IF(NCF.EQ.3)WRITE(10,2640)

2600 FORMAT(/' Choice of Green Function:')

2610 FORMAT(' Complete Homogeneous Full-Space Green Function')

2620 FORMAT(' Far-Field P & S waves Term (=1/r Term)')

2630 FORMAT(' Far-Field S waves Term (=1/r Term)')

2640 FORMAT(' Far-Field P waves Term (=1/r Term)')

C

C *** CHANGE OF SIGNS FOR IMAGINARY PARTS ***

C

read(7,*)

read(7,*)

READ(7,*)ISIGN

c      write(*,*)ISIGN

IF(ISIGN.EQ.1)WRITE(10,2700)

IF(ISIGN.EQ.0)WRITE(10,2710)

2700 FORMAT(/' SIGNS OF IMAGINARY PARTS FOR RESULTS ARE CHANGED')

2710 FORMAT(/' SIGNS OF IMAGINARY PARTS FOR RESULTS ARE NOT CHANGED')

C

C *** OP DATA ***

C

read(7,*)

read(7,*)

READ(7,*)NOB

c      write(*,*)NOB

WRITE(10,2800)NOB

2800 FORMAT(//' TOTAL NUMBER OF OBSERVATION POINTS =',I6/)

C

IF(NOB.GT.MOB)THEN

WRITE(10,*)' NOB>NMOB !!!!!!!!!!!!!'

STOP

END IF

C

read(7,*)

DO 300 I=1,NOB

READ(7,*)j,XO(I),YO(I),ZO(I)

write(*,*)j,XO(I),YO(I),ZO(I)

WRITE(10,2820)I,XO(I),YO(I),ZO(I)

300 CONTINUE

2820 FORMAT('   OB NUM.=',I5,',     (X,Y,Z) =',3F12.4)

c

CLOSE(UNIT=7)

c

RETURN

END

c

c

c *** Randam Number with a normally distributed deviate with zero mean

c     and unit variance, using ran0 as the source of uniform deviates

c

関数

・正規分布のランダム数

function gasdev(idum)

IMPLICIT REAL*8(A-H,O-Z)

save iset,gset

data iset/0/

if(iset.eq.0)then

10   v1=2.d0*ran0(idum)-1.d0

v2=2.d0*ran0(idum)-1.d0

rsq=v1*v1+v2*v2

if(rsq.ge.1.d0.or.rsq.eq.0.d0)goto 10

fac=dsqrt(-2.d0*dlog(rsq)/rsq)

gset=v1*fac

gasdev=v2*fac

iset=1

else

gasdev=gset

iset=0

end if

return

end

関数

・ランダム数発生

c *** Randam Function **

c

function ran0(idum)

IMPLICIT REAL*8(A-H,O-Z)

parameter(IA=16807,IM=2147483647,AM=1.d0/IM,IQ=127773

*         ,IR=2836,MASK=123459876)

c

idum=ieor(idum,MASK)

k=idum/IQ

idum=IA*(idum-k*IQ)-IR*k

if(idum.lt.0)idum=idum+IM

ran0=AM*idum

idum=ieor(idum,MASK)

c

return

end

c

c

c *** Calculation of Normalized Boore's Moment Rate Functions

c     (Far Field Displacement) at the all sub-faults ***

c

SUBROUTINE Boore(MOM,MFL,Frate,rate)

C

IMPLICIT REAL*8(A-H,O-Z)

COMPLEX*16 FX(MOM),FX1(MOM),cran,Frate(MOM,MFL,2)

c      COMPLEX*16 FX(MOM),Frate(MOM,MFL,2),cran1(MOM)

DIMENSION XR(MOM),bov(mom),bovN(mom),rate(mom,MFL,2),bos(mom)

c      DIMENSION XR(MOM),rate(mom,MFL,2)

COMMON /FREQ/DOM,IOM,NOM /OBS/XO(51),YO(51),ZO(51),NOB

*    /time/dt,period /ntime/nt

サブルーチン

・Boore型震源モデル

・高振動数側はランダム位相を、低振動数側では０位相を用いる→この例題では外部データのランダム位相を使用

*    /FLT/STR,DIP,RAK(203),FL,FW,FD(203),VR,TLc,Pn(203)

*        ,XF(203),YF(203),TD(203),DP(203),fc(203),fmax(203)

*    /Radiation/fR1,fR2,fran

*    /IFLT/NF,NSTR,NDIP,NCF,NFfunc /Nboore/Niter,idum

c

nc=51

if(nc.lt.NOB)then

write(10,*)'from Boore:'

WRITE(10,*)' NC < NOB !!!!!!!!!!!!!'

write(*,*)'from Boore:'

WRITE(*,*)' NC < NOB !!!!!!!!!!!!!'

STOP

END IF

c

nc=203

if(nc.lt.NF)then

write(10,*)'from Indat:'

WRITE(10,*)' NC < NF !!!!!!!!!!!!!'

write(*,*)'from Indat:'

fran：ランダム位相を用いる振動数の上限→ここではfran=0で、全振動数でランダム位相を使用

Niter：ω2モデルに震源スペクトルへの収束計算数

idum：ランダム位相発生のための初期値

WRITE(*,*)' NC < NF !!!!!!!!!!!!!'

STOP

END IF

c

c   * Input Data *

c

read(7,*)

read(7,*)

read(7,*)fran,Niter,idum

write(10,*)

write(10,*)'From subroutine Boore:'

write(10,*)'  Upper Limit Freq. for Random Phases (Hz) ', Fran

write(10,*)'  Number of Iterrations =',Niter

write(10,*)'  Initial Random Number (1-999) =',idum

c

c   * Initial data

c

pi=datan(1.d0)*4.d0

dur=dt*dfloat(nt)

nfast=nom*2

c

c   * Spectra of Nomalized Moment Rate Function *

c

write(*,*)'  One Example of Phase Spectra is stored in phase.csv'

open(21,file='Source(No.1)-phase.csv')

write(21,*)'freq.(Hz),','phase,','envelope(+),','envelope(-)'

c

write(*,*)'  Now calculating moment-rates for all the sub-faults!'

do 100 iflt=1,NF

write(*,*)' Fault No. =',iflt

c

c   * Boore's Envelope Function *

c     (Tdu: the source duration (see eq.(6) in Boore (1983))

c

Tdu=1.d0/fc(iflt)

Tw=2.d0*Tdu

b=1.2531d0

c=5.d0*b/Tw

a=(5.d0*2.718281829d0/Tw)**b

bov(1)=0.d0

bovN(1)=0.d0

Tdu2=Tdu*2.d0

Tdu3=Tdu*3.d0

pi=datan(1.d0)*4.d0

write(*,*)'Boore Envelope Function is stored in boore-env.csv'

open(20,file='boore-env.csv')

write(20,*)'time,','boore,','envelope'

do i=2,nt

bov(i)：Boore型経時関数

time=dt*(i-1)

bov(i)=a*(time**b)*dexp(-c*time)

if(time.gt.Tdu2)then

if(time.le.Tdu3)then

bov(i)=bov(i)*dcos(pi*(time-Tdu2)/2.d0/Tdu)

else

時間が3*Tdu(=3/fc)以降に振幅が0になるようにcos関数で調整

bov(i)=0.d0

end if

end if

bovN(i)=bov(i)**(1.d0/dfloat(Niter))

if((bovN(i).lt.1.d-10).or.(Niter.eq.0))bovN(i)=0.d0

write(20,9999)time,bov(i),bovN(i)

bovN(i)：bov**(1/Niter)として収束回数(Niter)後にbovとなるように調整した経時関数

end do

close(20)

9999   format(f12.4,5(',',e15.7))

c

DO 110 irnd=1,2

c

DO 120 IO=1,nom+1

OM=DOM*DFLOAT(IO-1)

f=om/(2.d0*pi)

c     * Brune's omega-square model *

FFc=f/Fc(iflt)

S=1.d0/(1.d0+FFc*FFc)

c     * Fmax by Boore (1983) n=4 *

FFmax=f/Fmax(iflt)

P=1.d0/dsqrt(1.d0+FFmax**(2.d0*Pn(iflt)))

c     * Fmax by Kamae et al. (1991) *

c            P=1.d0/(1.d0+FFmax)

SA=S*P

bos(io)：ω2モデル

（振幅スペクトル）

c     * Double derivatives of Moment Rate Function *

c            FX(io)=SA*cran1(io)*(-om*om)

bos(io)=SA*om*om

c

c     * Generation of Phases (zero for lower and random for higher freq.)*

ran=(ran0(idum)-0.5d0)*pi*2.d0

phsav=pi

if(f.lt.Fran)then

Franより低振動数は徐々に0位相にする

→ここでは使用せず

amp=f/Fran

amp=dsin(amp*pi/2.d0)

amp=amp*amp

amp=amp*amp

ran=ran*amp

phsav=phsav*amp

end if

dr=+dcos(ran)

di=-dsin(ran)

cran：ランダム位相

cran=dcmplx(dr,di)

if((iflt.eq.1).and.(irnd.eq.1))then

write(21,*)f,',',ran,',',phsav,',',-phsav

end if

c

c     * Double Derivative of Moment Rate Function (Far-Field Acceleration) *

FX(io)=SA*cran*(-om*om)

FX1(io)=FX(io)

FX(io)：ω2モデルの震源スペクトル

c

120   CONTINUE

c

c     * Calculating First Waveforms (accelaration) using Boore's envelop funtion *

c

DO I=2,NFAST/2

FX(NFAST+2-I)=dCMPLX(dREAL(FX(I)),-DIMAG(FX(I)))

end do

CALL dWVFAST(NFAST,FX,MOM,1)

震源時間関数（フーリエ逆変換による波形計算、初期値としてbovによる経時関数）

DO I = 1,NFAST

XR(I)=dREAL(FX(I))/DUR

XR(i)=XR(i)*bov(i)

end do

c

c     * Iterations to obtain Boore's waveform functions *

c

do 150 iter=1,Niter

c

収束計算（Niter回数）：

震源時間関数のフーリエ変換と、震源スペクトルをω２スペクトルで振幅調整

c       * Correction of Fourier Amplitude *

do i=1,nfast

FX(I)=dCMPLX(XR(I),0.0d0)

end do

CALL dWVFAST(nfast,FX,MOM,-1)

DO K=1,nom+1

FX(K)=FX(K)*dt

FX(K)=FX(K)*cdabs(FX1(K))/cdabs(FX(K))

end do

c

c       * Waveform (accelaration) *

DO I=2,NFAST/2

FX(NFAST+2-I)=dCMPLX(dREAL(FX(I)),-DIMAG(FX(I)))

end do

CALL dWVFAST(NFAST,FX,MOM,1)

震源スペクトルのフーリエ逆変換と、bovNで経時関数の振幅調整

DO 180 I = 1,NFAST

XR(I)=dREAL(FX(I))/DUR

XR(i)=XR(i)*bovN(i)

180     CONTINUE

c

150   CONTINUE

c

c   * Double derivatives of Momant Rate Function (Far Field acceleration) *

c

rate：震源時間関数の２階時間微分（遠方場での加速度波形相当）

DO I = 1,NFAST

rate(i,iflt,irnd)=xr(i)

end do

c

c   * Fourier Amplitude of Double derivatives of Momant Rate Function *

c

do i=1,nfast

FX(I)=dCMPLX(rate(i,iflt,irnd),0.0d0)

end do

CALL dWVFAST(nfast,FX,MOM,-1)

DO K=1,nom+1

Frate：rateのフーリエスペクトル

om=dom*(k-1)

f=om/(2.d0*pi)

Frate(K,iflt,irnd)=dconjg(FX(K))*dt

end do

c

110   continue

c

100 continue

close(21)

c

c   * Data Output *

c

write(*,*)

write(*,*)'  Two sets of the double derivatives of the normalized'

write(*,*)'  moment rates are stored in'

write(*,*)'  Moment-Rate-Acc-1.csv and Moment-Rate-Acc-2.csv'

open(22,file='Moment-Rate-Acc-1.csv')

write(22,2000)(iflt,iflt=1,NF)

do i=1,nfast

time=dt*(i-1)

write(22,2001)time,(rate(i,iflt,1),iflt=1,NF)

end do

close(22)

open(22,file='Moment-Rate-Acc-2.csv')

write(22,2000)(iflt,iflt=1,NF)

do i=1,nfast

time=dt*(i-1)

write(22,2001)time,(rate(i,iflt,2),iflt=1,NF)

end do

close(22)

2000 format('time(sec)',300(',Sub-Fault No.',I3))

2001 format(f8.4,300(',',e17.5))

c

write(*,*)'  Two-sets of Fourier amp. of moment-rates are stored'

write(*,*)'  in Moment_Rate-Amp-1.csv and Moment_Rate-Amp-2.csv'

open(23,file='Moment-Rate-AccAmp-1.csv')

write(23,2100)(iflt,iflt=1,NF)

DO K=1,nom+1

om=dom*(k-1)

f=om/(2.d0*pi)

c        write(23,2101)f,cdabs(fx1(k)),(cdabs(Frate(k,iflt,1)),iflt=1,NF)

write(23,2101)f,bos(k),(cdabs(Frate(k,iflt,1)),iflt=1,NF)

end do

close(23)

open(23,file='Moment-Rate-AccAmp-2.csv')

write(23,2100)(iflt,iflt=1,NF)

DO K=1,nom+1

om=dom*(k-1)

f=om/(2.d0*pi)

write(23,2101)f,bos(k),(cdabs(Frate(k,iflt,2)),iflt=1,NF)

end do

close(23)

2100 format('freq.(Hz),Theory for the last sub-fault',

*    300(',Sub-Fault No.',I3))

2101 format(f8.4,300(',',e15.7))

c

return

END

c

C

C *** DATA OUTPUT ***

C

サブルーチン

・結果出力

SUBROUTINE OTDAT(MOM,MOB,GX,GY,GZ)

C

IMPLICIT REAL*8(A-H,O-Z)

COMPLEX*16 GX(MOM,MOB),GY(MOM,MOB),GZ(MOM,MOB)

COMMON /FREQ/DOM,IOM,NOM /OBS/XO(51),YO(51),ZO(51),NOB

*    /SIGN/ISIGN

C

C *** CHECK ***

C

NC=51

IF(NOB.GT.NC)THEN

WRITE(10,*)'FROM OTDAT:NOB>NC',NOB,NC

STOP

END IF

C

C *** OUTPUT OF DATA (GX, GY AND GZ)***

C

IF(ISIGN.EQ.1)THEN

DO 100 IOB=1,NOB

DO 110 K=IOM,NOM

GX(K,IOB)=DCMPLX(DREAL(GX(K,IOB)),-DIMAG(GX(K,IOB)))

GY(K,IOB)=DCMPLX(DREAL(GY(K,IOB)),-DIMAG(GY(K,IOB)))

GZ(K,IOB)=DCMPLX(DREAL(GZ(K,IOB)),-DIMAG(GZ(K,IOB)))

110   CONTINUE

100   CONTINUE

END IF

C

OPEN(UNIT=8,FILE='grfault.axyz',STATUS='UNKNOWN')

WRITE(8,2500)DOM,IOM,NOM

WRITE(8,2510)NOB

DO 500 IOB=1,NOB

DO 510 K=IOM,NOM

WRITE(8,2520)GX(K,IOB),GY(K,IOB),GZ(K,IOB)

510     CONTINUE

500   CONTINUE

CLOSE(8)

C

2500 FORMAT(1X,F10.6,2I10)

2510 FORMAT(1X,I10,///)

2520 FORMAT(1X,6E12.5)

C

RETURN

サブルーチン・強震波形計算

END

C

C

C *** COMPUTATION OF GREEN FUNCTIONS of the Homogeneous Full-Space ***

C

SUBROUTINE HKQVW(MOM,MOB,MFL,Frate,GGX,GGY,GGZ)

C

IMPLICIT REAL*8(A-H,O-Z)

COMPLEX*16 VPc,VSc,SM,Frate(MOM,MFL,2),cpx

*      ,GGX(MOM,MOB),GGY(MOM,MOB),GGZ(MOM,MOB)

*      ,SMO,SM00,EDT,GX,GY,GZ,GXL,GYL,GZL

*      ,GXHp,GYHp,GZHp,GRHp,GXHs,GYHs,GZHs

*      ,GXHsh,GYHsh,GTHsh

*      ,UX(3,3),UY(3,3),UZ(3,3)

*      ,Ukij,kai,kair,fair,esr,epr

*      ,ss,sr,sr2,sr3,pp,pr,pr2,pr3,ps2

c     *      ,GXHsh,GYHsh,GTHsh,GXHsv,GYHsv,GZHsv,GRHsv

DIMENSION EN(3,3),rd(3)

COMMON /FREQ/DOM,IOM,NOM /OBS/XO(51),YO(51),ZO(51),NOB

*    /STRC/DNS,VP,QP,FP,VS,QS,FS /IFLT/NF,NSTR,NDIP,NCF,NFfunc

*    /FLT/STR,DIP,RAK(203),FL,FW,FD(203),VR,TLc,Pn(203)

*        ,XF(203),YF(203),TD(203),DP(203),fc(203),fmax(203)

*    /Radiation/fR1,fR2,fran

C

C *** CHECK OF DIMENSION ***

C

NC=51

IF(NOB.GE.NC)THEN

WRITE(10,*)' FROM HKQVW:NOB>NC !!!!!!!!!!'

WRITE(10,*)' NOB,NC=',NOB,NC

STOP

END IF

C

NC=203

IF(NF.GE.NC)THEN

WRITE(10,*)' FROM HKQVW:NF>NC !!!!!!!!!!'

WRITE(10,*)' NF,NC=',NF,NC

STOP

END IF

C

c *** Various Parameters ***

c

CPX=(0.D0,1.D0)

pai=4.d0*datan(1.d0)

CST= DCOS(STR)

SST= DSIN(STR)

CDP= DCOS(DIP)

SDP= DSIN(DIP)

c      CDP2=CDP*CDP

c      SDP2=SDP*SDP

c      CDP4= DCOS(4.d0*DIP)

c      CDP22=(1.d0+CDP4)/2.d0

c      SDP22=(1.d0-CDP4)/2.d0

c

c *** Homogeneous High-Freq. Radiation Coefficients for P and S waves ***

c

write(10,*)

FpH=dsqrt(4.d0/15.d0)

高振動数での等方放射係数

FsH=dsqrt(0.4D0)

WRITE(10,*)

write(10,*)'Radiation Coefficients'

WRITE(10,*)'Homogeneous Coefficients for P and S waves =',FpH,FsH

C

C *** GREEN'S FUNCTION ***

C

do iob=1,nob

do io=1,nom

GGX(IO,IOB)=(0.D0,0.D0)

GGY(IO,IOB)=(0.D0,0.D0)

GGZ(IO,IOB)=(0.D0,0.D0)

end do

end do

c

等方弾性体のグリーン関数を計算

c   * Green's function computations *

c

sNF=dsqrt(dfloat(NF))

DO 100 IO=IOM,NOM

C

OM=DOM*DFLOAT(IO)

freq=om/(2.d0*pai)

OMTd=OM*TLc

sinc=dsin(OMTd)/OMTd

write(*,1005)freq,dfloat(io)/dfloat(nom)*100.d0

1005   format('   Freq (%) =',f7.3,' (',f5.1,' %),')

c

DO 200 IOB=1,NOB

C

DO 300 IS=1,NF

c

if(FD(IS).eq.0.d0)goto 300

c

RX=XF(IS)-XO(IOB)

RY=YF(IS)-YO(IOB)

RZ=DP(IS)-ZO(IOB)

RR=DSQRT(RX*RX+RY*RY+RZ*RZ)

rd(1)=Rx/RR

rd(2)=Ry/RR

rd(3)=Rz/RR

CRK= DCOS(RAK(is))

SRK= DSIN(RAK(is))

c            CRK2=CRK*CRK

c            SRK2=SRK*SRK

c

EDT=CDEXP(cpx*OM*TD(IS))

c

c    * Complex VP, VS, and, Moment Rate Function *

振動数依存Q値

call QPQS(freq,QPe,QSe)

if(QP.eq.0.d0)then

VPc=VP*DCMPLX(1.D0,0.d0)

else

VPc=VP*DCMPLX(1.D0,-0.5D0/QPe)

end if

if(QS.eq.0.d0)then

VSc=VS*DCMPLX(1.D0,0.d0)

else

VSc=VS*DCMPLX(1.D0,-0.5D0/QSe)

end if

SM=DNS*VSc*VSc

c    * Seismic Moment *

SM00=FD(IS)*SM*FL*FW/sNF

c    * Slip Velocity Function -> Slip Displacement Function *

SMO=SM00*CPX/OM

c

c    * fai, kai, fai,r, and kai,r *

ss=+cpx*om/VSc

pp=+cpx*om/VPc

sr=ss*rr

pr=pp*rr

ps2=pp*pp/(ss*ss)

esr=cdexp(sr)/rr

epr=cdexp(pr)/rr

c

if(NCF.eq.0)then

c

c    * Complete Green's Functions of Homogeneous Full-Space *

c

sr2=sr*sr

sr3=sr*sr2

pr2=pr*pr

pr3=pr*pr2

kai=(3.d0/sr2-3.d0/sr+1.d0)*esr

kai=kai-ps2*(3.d0/pr2-3.d0/pr+1.d0)*epr

kair=(-9.d0/sr3+9.d0/sr2-4.d0/sr+1.d0)*esr*ss

kair=kair-ps2*(-9.d0/pr3+9.d0/pr2-4.d0/pr+1.d0)*epr*pp

fair=(-3.d0/sr3+3.d0/sr2-2.d0/sr+1.d0)*esr*ss

fair=fair-ps2*(-3.d0/pr3+3.d0/pr2-1.d0/pr)*epr*pp

c

else

c

c    * Green's Function of the far field S & P terms *

c

kai=(0.d0,0.d0)

if(NCF.eq.1)then

kair=esr*ss

kair=kair-ps2*epr*pp

fair=esr*ss

else

if(NCF.eq.2)then

kair=esr*ss

fair=esr*ss

else

if(NCF.eq.3)then

kair=(0.d0,0.d0)

kair=-ps2*epr*pp

fair=(0.d0,0.d0)

end if

end if

end if

c

end if

c

c    * Green's Functions: Uki,j=Uxi,j=UX(i,j) *

do i=1,3

D1i=0.d0

if(1.eq.i)D1i=1.d0

D2i=0.d0

if(2.eq.i)D2i=1.d0

D3i=0.d0

if(3.eq.i)D3i=1.d0

do j=1,3

D1j=0.d0

if(1.eq.j)D1j=1.d0

D2j=0.d0

if(2.eq.j)D2j=1.d0

D3j=0.d0

if(3.eq.j)D3j=1.d0

Dij=0.d0

if(i.eq.j)Dij=1.d0

rdij1=rd(i)*rd(j)*rd(1)

Ukij=fair*rd(j)*D1i-kair*rdij1

Ukij=Ukij-(rd(i)*D1j+rd(1)*Dij-2.d0*rdij1)*kai/RR

UX(i,j)=Ukij/(4.d0*pai*SM)

c

rdij2=rd(i)*rd(j)*rd(2)

Ukij=fair*rd(j)*D2i-kair*rdij2

Ukij=Ukij-(rd(i)*D2j+rd(2)*Dij-2.d0*rdij2)*kai/RR

UY(i,j)=Ukij/(4.d0*pai*SM)

c

rdij3=rd(i)*rd(j)*rd(3)

Ukij=fair*rd(j)*D3i-kair*rdij3

Ukij=Ukij-(rd(i)*D3j+rd(3)*Dij-2.d0*rdij3)*kai/RR

UZ(i,j)=Ukij/(4.d0*pai*SM)

end do

Ukij：等方弾性体のグリーン関数のk方向微分（=Uij,k）

end do

c

c    * Low Frequency Wave *

c

XN=-SDP*SST

YN= SDP*CST

ZN=-CDP

EX= CRK*CST+SRK*CDP*SST

EY= CRK*SST-SRK*CDP*CST

EZ=-SRK*SDP

C

EN(1,1)= 2.D0*EX*XN

EN(1,2)= EX*YN +EY*XN

EN(1,3)= EX*ZN +EZ*XN

EN(2,1)= EY*XN +EX*YN

EN(2,2)= 2.D0*EY*YN

EN(2,3)= EY*ZN +EZ*YN

EN(3,1)= EZ*XN +EX*ZN

EN(3,2)= EZ*YN +EY*ZN

EN(3,3)= 2.D0*EZ*ZN

GXL=(0.D0,0.D0)

GYL=(0.D0,0.D0)

GZL=(0.D0,0.D0)

DO I=1,3

低振動数の強震波形

（理論放射係数を使用）

→ここでは使用せず

DO J=1,3

GXL=GXL+EN(I,J)*UX(J,I)

GYL=GYL+EN(I,J)*UY(J,I)

GZL=GZL+EN(I,J)*UZ(J,I)

end do

end do

c

c    * High Frequency Wave *

c

c      * High Frequency P Wave (Explosive Source) *

c

GXHp=(0.D0,0.D0)

GYHp=(0.D0,0.D0)

GZHp=(0.D0,0.D0)

if(NCF.eq.2)goto 555

c

高振動数の強震波形

（等方放射係数を使用）

→P波の膨張収縮震源

do i=1,3

GXHp=GXHp+UX(i,i)

GYHp=GYHp+UY(i,i)

GZHp=GZHp+UZ(i,i)

end do

c

c          * Unit Vector of the SP-OP line *

XN=-rd(1)

YN=-rd(2)

XY2=dsqrt(XN*XN+YN*YN)

XNS=XN/XY2

YNS=YN/XY2

c

c          * Radial Component and Distribution to X and Y components *

GRHp= XNS*GXHp+YNS*GYHp

P波を３成分に配分（放射係数を乗じ、（X,YはR成分を等配分）

GXHp=FpH*GRHp/dsqrt(2.d0)

GYHp=GXHp

GZHp=FpH*GZHp

c

c      * High Frequency SH Wave (Fault Plane is perpendicular to the OP-SP line) *

c

555        GXHsh=(0.D0,0.D0)

GYHsh=(0.D0,0.D0)

c             GXHsv=(0.D0,0.D0)

c             GYHsv=(0.D0,0.D0)

c             GZHsv=(0.D0,0.D0)

if(NCF.eq.3)goto 666

c

c      *

c          * Unit Vector of the Fault Plane *

XN=-rd(1)

YN=-rd(2)

ZN=-rd(3)

XY2=dsqrt(XN*XN+YN*YN)

XNS=XN/XY2

YNS=YN/XY2

C

c      * High Frequency SH Wave *

c          * Unit Slip Vector of the SH Source *

EX= YNS

EY=-XNS

EZ= 0.d0

c

c      * (Ei x Nj + Ej x Ni) and SH wave Green function *

c

EN(1,1)= 2.D0*EX*XN

EN(1,2)= EX*YN +EY*XN

EN(1,3)= EX*ZN +EZ*XN

EN(2,1)= EY*XN +EX*YN

EN(2,2)= 2.D0*EY*YN

EN(2,3)= EY*ZN +EZ*YN

EN(3,1)= EZ*XN +EX*ZN

EN(3,2)= EZ*YN +EY*ZN

EN(3,3)= 2.D0*EZ*ZN

c

高振動数の強震波形

（等方放射係数を使用）

→S波はSH震源のみ

DO I=1,3

DO J=1,3

GXHsh=GXHsh+EN(I,J)*UX(J,I)

GYHsh=GYHsh+EN(I,J)*UY(J,I)

end do

end do

c

c      * Transverse Component and Distribution to X and Y components *

GTHsh=-YNS*GXHsh+XNS*GYHsh

GXHs=FsH*GTHsh/dsqrt(2.d0)

SH波を３成分に配分（放射係数を乗じ、X,YはT成分を等配分）

GYHs=GXHs

GZHs=(0.d0,0.d0)

c

c    * Hybrid Wave *

c

666        if(freq.lt.fR1)then

GX=GXL

GY=GYL

GZ=GZL

else

if(freq.lt.fR2)then

dfreq=(freq-fR1)/(fR2-fR1)

GX=GXL+(GXHp+GXHs-GXL)*dfreq

GY=GYL+(GYHp+GYHs-GYL)*dfreq

GZ=GZL+(GZHp+GZHs-GZL)*dfreq

低振動数と高振動数の波形の合成（ハイブリッド）

→ここでは高振動数のみ

else

GX=GXHp+GXHs

GY=GYHp+GYHs

GZ=GZHp+GZHs

end if

end if

c

c    * Irikura's F-function *

if(NFfunc.eq.0)then

EDT=EDT*(1.d0+(sNF-1.d0)*sinc*exp(cpx*OMTd))

c    * Exp-type F-function *

else

EDT=EDT*(1.d0+(sNF-1.d0)/(1.d0-cpx*OMTd))

end if

c

GX=GX*SMO*EDT*Frate(io+1,is,1)

GY=GY*SMO*EDT*Frate(io+1,is,2)

GZ=GZ*SMO*EDT*(Frate(io+1,is,1)+Frate(io+1,is,2))/2.d0

GGX(IO,IOB)=GGX(IO,IOB)+GX

GGY(IO,IOB)=GGY(IO,IOB)+GY

io振動数、iob観測点への強震波形の代入（X,Y成分は異なるランダム位相、Z成分はX,Y位相の平均値）

GGZ(IO,IOB)=GGZ(IO,IOB)+GZ

c

300     CONTINUE

C

200   CONTINUE

C

100 CONTINUE

c

RETURN

END

サブルーチン・フーリエ変換（大崎先生の本より）

c

c

C

C *** FOURIER TRANSFORM OR INVERSE FOURIER TRANSFORM

C

SUBROUTINE dWVFAST(N,A,MOM,IND)

C

C *** N   : NUMBER OF FOURIER COEFF. OR WAVE ( = 2**I )

C *** A(I): FOURIER COEFFICIENTS ( N/2+1 ) OR WAVE ( REAL PART )

C    ( FOURIER SPECTRUM = FOURIER COEFFICIENTS * DURATION/2.0 )

C *** MOM : N IN MAIN DIMMENSION

C         : IF MWV > 2**13(=8192), CAN NOT USE !!!

C *** IND : = -1 : TRANS.,   = 1 : INVERSE TRANS.

C

IMPLICIT REAL*8(A-H,O-Z)

COMPLEX*16 A(MOM),TEMP,THETA

C

C *** CALUCURATION OF FFT

C

PAI=3.1415926540d0

J=1

DO 140 I=1,N

IF(I.GE.J) GO TO 110

TEMP=A(J)

A(J)=A(I)

A(I)=TEMP

110   M=N/2

120   IF(J.LE.M) GO TO 130

J=J-M

M=M/2

IF(M.GE.2) GO TO 120

130   J=J+M

140 CONTINUE

KMAX=1

150 IF(KMAX.GE.N)RETURN

ISTEP=KMAX*2

DO 170 K=1,KMAX

THETA=dCMPLX(0.d0,PAI*dFLOAT(IND*(K-1))/dFLOAT(KMAX))

DO 160 I=K,N,ISTEP

J=I+KMAX

TEMP=A(J)*CdEXP(THETA)

A(J)=A(I)-TEMP

A(I)=A(I)+TEMP

160   CONTINUE

170 CONTINUE

KMAX=ISTEP

GO TO 150

C

END

c

c

サブルーチン

・振動数依存Q値

c *** Quality Factors of Vp and Vs ***

c

subroutine QPQS(freq,QPe,QSe)

c

IMPLICIT REAL*8 (A-H,O-Z)

COMMON /STRC/DNS,VP,QP,FP,VS,QS,FS

c      if(freq.eq.0.d0)then

c        QPe=QP

c        QSe=QS

c      else

QPe=QP*freq**FP

if(QPe.lt.QP)QPe=QP

QSe=QS*freq**FS

if(QSe.lt.QS)QSe=QS

c      end if

c

return

end