*** grflt01d.f (2005, Aug.) ***

This code modified from grflt01p.f to use Green's function of a static homogeneous full-space  (direct wave) as an aymptotic solution, but not Hisada's solution (1995).

*** grflt01b.f (2005, Aug.) ***

This code combined grflt01p.f and boore5.f to use Boore’s stochastic source model and Green's function of homogeneous half-spaces.

--> This code has a problem, which has the problem as the same as Boore7

***  Boore6.f ***

This program was modified from boore5.f to correct the reduction of amplitudes due to phase differences caused by Vr and Vs among sub-faults. (Aug., 2005)

--> This code has a problem, which introduces artificial dominant frequencies, due to regularly distributed source points.

***  Boore7.f ***

This program was modified from boore6.f to correct the reduction of amplitudes due to phase differences caused by Vr and Vs among sub-faults using the following techniques.

- The amplification factor for Vr and Vs are used only at coherent (low) frequencies (f < fran). At random (high) frequency range (f > fran), the amplification factors for numbers of sub-faults (sqrt(NL*NW) are used.

- In order to remove the artificial dominant frequencies, random distributions of rupture times are introduced.

- The amplification factor for Vr and Vs are smoothed out to avoid extremely large amplitudes, by using the moving average method. (Aug., 2005)

*** grflt02b.f (2005, Aug.) ***

This code combined grflt01p.f and boore7.f to use Boore’s stochastic source model and Green's function of homogeneous half-spaces.

--> This code has a problem, in which the layered structures are not used at high frequencies.

***  Boore8.f ***

This program was modified from boore7.f to introduce the frequency dependent fault angles for radiation pattern. The fault angles are deviated using the normal Gaussian distribution.

-->  This code has a problem. For high frequencies, the simulated waves are completely random waves, because of the introduction of frequency dependent random angles. The deviated fault angles should be independent of frequencies (Aug.,2005).

***  Boore9.f ***

This program was modified from boore7.f to introduce the homogeneous radiation pattern at high frequency. A P-wave source (expansion/compression  source) and a SH shear source for S waves are used to generate pure P and S waves, respectively. After homogenous amplitude factors (0.52 and 0.63 for P and S, respectively) are multiplied, the P and S Waves of one component are divided into 3 components (Aug.,2005).

***  Boor10.f ***

This program was modified from boore9 (Aug.,2005). The high frequency P is the same as boore9.f, but high frequency S is modified, where only horizontal (SH) waves are generated and distributed into two components (X and Y components).

-> For the 2005 SSJ, this program + milti3.f are used.

***  Grflr03b.f ***

This code combined grflt01p.f and boore9.f to use Boore’s stochastic source model and Green's function of homogeneous half-spaces.

***  Grflr04b.f ***

This code combined grflt01p.f and boor10.f to use Boore’s stochastic source model and Green's function of homogeneous half-spaces.

-> For the 2005 SSJ, this program + phs3.f + grfftp.f are used.