FFT、微積分、及びフィルタープログラム

（プログラム：ftp://kansoku.cc.kogakuin.ac.jp/pub/hisada/fft/tran.f）

（データ：ftp://kansoku.cc.kogakuin.ac.jp/pub/hisada/fft/t068.acc）

C

PROGRAM tran

C

PARAMETER(MWV=33000)

CHARACTER INFILE*20

REAL time(MWV)

REAL XR1(MWV),XR2(MWV),XR3(MWV),X1(MWV),X2(MWV),X3(MWV)

COMPLEX*8 FX1(MWV),FX2(MWV),FX3(MWV)

C

WRITE(*,*)

WRITE(*,*) 'FFT and Band-Pass Filter for Taiwan Data'

WRITE(*,*) 'Read from "***.acc" '

WRITE(*,*) '------------------------------------------------------

+-----------'

WRITE(*,*)

C

C  Prompt for input RECON'ed file

C

WRITE(*,1010)

1010 FORMAT(1X,'Enter name "***.acc')

READ(*,1020) INFILE

1020 FORMAT(A20)

C

C *** Read input data file  ***

C

OPEN(11,FILE=INFILE)

i=1

10   read(11,*,end=11)time(i),xr1(i),xr2(i),xr3(i)

i=i+1

goto 10

11 CLOSE(11)

c

NT=i-1

dt=time(2)-time(1)

write(*,*)'NT and DT=',NT,DT

C

C *** FOURIER TRANSFORM ***

C

NF=2

DO 200 I=2,16

NF=2*NF

IF(NF.GE.NT)GOTO 30

200 CONTINUE

WRITE(*,*)' NT>2**16 !!!!!'

STOP

30 IF1=I

WRITE(*,*)

WRITE(*,*)' IF,NF=',IF1,NF

C

DO 300 I=1,NT

FX1(I)=CMPLX(XR1(I),0.0)

FX2(I)=CMPLX(XR2(I),0.0)

FX3(I)=CMPLX(XR3(I),0.0)

300 CONTINUE

DO 310 I=NT+1,NF

FX1(I)=(0.0,0.0)

FX2(I)=(0.0,0.0)

FX3(I)=(0.0,0.0)

310 CONTINUE

c

CALL WVFAST(NF,FX1,MWV,-1)

CALL WVFAST(NF,FX2,MWV,-1)

CALL WVFAST(NF,FX3,MWV,-1)

DUR=FLOAT(NF)*DT

DO 400 I=1,NF

FX1(I)=FX1(I)*DT

FX2(I)=FX2(I)*DT

FX3(I)=FX3(I)*DT

400 CONTINUE

c

c      FOM=0.0

DOM=6.2831853070e0/DUR

NOM=NF/2+1

EOM=DOM*NOM

WRITE(*,*)

WRITE(*,*)' DUR,DOM,NOM,EOM=',DUR,DOM,NOM,EOM

C

C *** BAND PATH ***

C

WRITE(*,*)

WRITE(*,*)' BAND PASS? (YES=1)?? :'

READ(*,*)NBD

C

IF(NBD.EQ.1)THEN

C

WRITE(*,*)

WRITE(*,*)'  TRAPEZOIDAL BAND PATH FILTER'

c        WRITE(*,*)'  INITIAL AND FINAL OM=',DOM*IOM,DOM*(NDATA-1)

WRITE(*,*)'  ENTER OML1<OML2<OMH1<OMH2'

READ(*,*)OML1,OML2,OMH1,OMH2

C

DO 500 I = 1,NOM

OMEGA=FLOAT(I-1)*DOM

IF(OMEGA.LE.OML1)then

FX1(I)=(0.0,0.0)

FX2(I)=(0.0,0.0)

FX3(I)=(0.0,0.0)

end if

IF(OMEGA.GE.OMH2)then

FX1(I)=(0.0,0.0)

FX2(I)=(0.0,0.0)

FX3(I)=(0.0,0.0)

end if

AA=1.0

IF(OMEGA.LT.OML2)AA=(OMEGA-OML1)/(OML2-OML1)

IF(OMEGA.GT.OMH1)AA=(OMH2-OMEGA)/(OMH2-OMH1)

FX1(I) = AA*FX1(I)

FX2(I) = AA*FX2(I)

FX3(I) = AA*FX3(I)

500   CONTINUE

C

END IF

C

WRITE(*,*)

WRITE(*,*)' Integ.(=1), Double Integ.(=2) or Diff.(=3) ?:'

READ(*,*)NIT

c

IF(NIT.EQ.1)THEN

DO 550 I = 2,NOM

OMEGA=FLOAT(I-1)*DOM

FX1(I) = FX1(I)/cmplx(0.0,omega)

FX2(I) = FX2(I)/cmplx(0.0,omega)

FX3(I) = FX3(I)/cmplx(0.0,omega)

550   CONTINUE

END IF

c

IF(NIT.EQ.2)THEN

DO 560 I = 2,NOM

OMEGA=FLOAT(I-1)*DOM

FX1(I) = FX1(I)/(-omega*omega)

FX2(I) = FX2(I)/(-omega*omega)

FX3(I) = FX3(I)/(-omega*omega)

560   CONTINUE

END IF

c

IF(NIT.EQ.3)THEN

DO 570 I = 2,NOM

OMEGA=FLOAT(I-1)*DOM

FX1(I) = FX1(I)*cmplx(0.0,omega)

FX2(I) = FX2(I)*cmplx(0.0,omega)

FX3(I) = FX3(I)*cmplx(0.0,omega)

570   CONTINUE

END IF

c

c * Output Fourier Spectrum data *

c

write(*,*)

write(*,*)'The Fourier Spectrum data are stored in "amp_dat"'

open(10,file='amp_dat',status='unknown')

DO 680 I = 2,NOM

om=FLOAT(I-1)*DOM

fr=om/6.283185307

write(10,1900)om,fr,cabs(FX1(I)),cabs(FX2(I)),cabs(FX3(I))

680 CONTINUE

1900 format(f8.2,f8.3,3e12.4)

close(10)

C

C *** INVERSE FOURIER TRANSFORM  ***

C

DO 600 I=2,NF/2

FX1(NF+2-I)=CMPLX(REAL(FX1(I)),-AIMAG(FX1(I)))

FX2(NF+2-I)=CMPLX(REAL(FX2(I)),-AIMAG(FX2(I)))

FX3(NF+2-I)=CMPLX(REAL(FX3(I)),-AIMAG(FX3(I)))

600 CONTINUE

C

CALL WVFAST(NF,FX1,MWV,1)

CALL WVFAST(NF,FX2,MWV,1)

CALL WVFAST(NF,FX3,MWV,1)

C

C *** WAVE DATA ***

C

DO 700 I = 1,NF

time(I) = FLOAT(I-1)*DT

XR1(I)=REAL(FX1(I))/DUR

XR2(I)=REAL(FX2(I))/DUR

XR3(I)=REAL(FX3(I))/DUR

700 CONTINUE

c

c *** Cut ***

c

do i=1,nt

x1(i)=xr1(i)

x2(i)=xr2(i)

x3(i)=xr3(i)

end do

write(*,*)

write(*,*)'Cut data? (No=0,Yes=1)'

read(*,*)icut

if(icut.eq.1)then

write(*,*)'Enter Starting Time (sec)'

read(*,*)stime

nstime=stime/dt

write(*,*)'Number of Cutting Data =',nstime

write(*,*)'Enter Duration (sec)'

read(*,*)dur

ndur=dur/dt+1

write(*,*)'Number of data =',ndur

do i=1,ndur

if((i+nstime).le.0)then

x1(i)=0.0

x2(i)=0.0

x3(i)=0.0

else

x1(i)=xr1(i+nstime)

x2(i)=xr2(i+nstime)

x3(i)=xr3(i+nstime)

end if

end do

nt=ndur

write(*,*)'New nt =',nt

end if

C

c *** output ***

c

write(*,*)

write(*,*)'Skip Data (no=0; yes=skip number)?'

read(*,*)iskip

iskip=iskip+1

WRITE(*,*)'wave2_dat:'

OPEN(11,FILE='wave2_dat',STATUS='unknown')

do i=1,nt,iskip

write(11,2000)dt*(i-1),X1(I)-x1(1),x2(i)-x2(1),x3(i)-x3(1)

end do

2000 format(4e12.4)

CLOSE(11)

c

STOP

END

C

C *** FOURIER TRANSFORM OR INVERSE FOURIER TRANSFORM

C

SUBROUTINE WVFAST(N,A,MOM,IND)

C

C *** N   : NUMBER OF FOURIER COEFF. OR WAVE ( = 2**I )

C *** A(I): FOURIER COEFFICIENTS ( N/2+1 ) OR WAVE ( REAL PART )

C    ( FOURIER SPECTRUM = FOURIER COEFFICIENTS * DURATION/2.0 )

C *** MOM : N IN MAIN DIMMENSION

C         : IF MWV > 2**13(=8192), CAN NOT USE !!!

C *** IND : = -1 : TRANS.,   = 1 : INVERSE TRANS.

C

COMPLEX*8 A(MOM),TEMP,THETA

C

C *** CALUCURATION OF FFT

C

PAI=3.1415926540e0

J=1

DO 140 I=1,N

IF(I.GE.J) GO TO 110

TEMP=A(J)

A(J)=A(I)

A(I)=TEMP

110   M=N/2

120   IF(J.LE.M) GO TO 130

J=J-M

M=M/2

IF(M.GE.2) GO TO 120

130   J=J+M

140 CONTINUE

KMAX=1

150 IF(KMAX.GE.N) RETURN

ISTEP=KMAX*2

DO 170 K=1,KMAX

THETA=CMPLX(0.0,PAI*FLOAT(IND*(K-1))/FLOAT(KMAX))

DO 160 I=K,N,ISTEP

J=I+KMAX

TEMP=A(J)*CEXP(THETA)

A(J)=A(I)-TEMP

A(I)=A(I)+TEMP

160   CONTINUE

170 CONTINUE

KMAX=ISTEP

GO TO 150

C

END