Application of  Grflto2x (High-Freq Method, Excel-Ver.) programs:

1994 Northridge Simulation using the Wald Model (1996)

Aug. 25, 2006

0. References

a. The source data of the 1994 Northridge earthquake model

http://pasadena.wr.usgs.gov/office/wald/Landers/bssa.html

http://pasadena.wr.usgs.gov/office/wald/slip_models.html#Landers

We used the source data of Wald and Heaton (Strong Motion) to reproduce the strong motion at the Lucerne Valley Station.

b. Simulation results using this program

Not available yet.

c. Theories of this program

http://kouzou.cc.kogakuin.ac.jp/Member/Boss/Paper/95cgf/cgf.html   for Green’s function.

http://kouzou.cc.kogakuin.ac.jp/Member/Boss/Paper/2005/JEEA2004.pdf  for Phase Spectra (in Japansese)

Paper for the detail theory is not yet available.

1. Input Data

a. Rock Site Model:  grflto2-rock.csv  (Data contains 4 stations, U55, U56, SCS7, and SSU)

b. Sediment Site Model: grflto2-soil.csv (Data contains 11 stations, NHL, SCSe, SYL, RRS, SVA, VNY, U03, SHR, TAR, U53, and U55)

See Figure 1 for the fault model of 1994 Northridge earthquake and the observation stations

2. Run Programs

Figure 2: The fault model of 1994 Northridge earthquake and the observation stations

(a) the observation stations and fault plane                           (b) final slip distribution

a. Compile and run phs3sQx.f to obtain phase velocities of surface waves.

Enter grflto2x-rock.csv for the rock model, or grflto2x-soil. csv for the sediment model.

Enter 0, if you don’t check secular function and eigen vectors of surface waves.

Enter 0.01 or smaller number. Generally the smaller number gives the more accurate results.

Enter 200 or larger number. Generally the larger number gives the more accurate results.

Enter 1000 or larger number. Generally the larger number gives the more accurate results.

To plot phase and group velocities of the Love and Rayleigh waves, use the output csv files, Ldisper.csv and Rdisper.csv, respectively. You can also plot the medium response of the Love and Rayleigh waves, use the output csv files, Lmedres.csv and Rmedres.csv, respectively. See Figure 2, for examples for Love wave of the soil model.

Figure 2: the dispersion curves (left) and the medium response (right) of the Love wave

b. Compile and run grflto2x.f to obtain strong motions in the frequency domain.

See Figure 3 for the coordinate system and fault parameters.

Enter grflto2x-soil.csv for the rock model, or grflto2x-rock. csv for the sediment model.

Rake Angle

X (North)

Strike Angle

Y (East)

Z (Down)

Free Surface

Hypocenter

Length (L)

Width (W)

Rupture Front

Rupture Velocity (Vr)

X (Source Point)

Y (Observation Station)

Slip Direction

Figure 3: Coordinate system

and fault parameters

Dip Angle

Green’s Function

Fault Origin

1

2

3

4

Ns

Ns+1

Ns+2

2Ns

3Ns

NsNd

2Ns+1

Ns-1

5

Ns(Nd-1)+1

Ns(Nd-1)

Strike Direction

Strike Direction

c. Compile and run grfftspx.f to obtain strong motions in the time domain.

The Fourier amplitude data are stored in amp.csv, and the waveform data are stored in wave.csv.

Enter 1, when you plot waves in the X, Y, and Z coordinate system

Choose an observation station number (In this case, Enter 1)

Enter 1, when you change signs of amplitude. Otherwise, enter 0.

Enter 1, when you change the sign of the amplitude. Otherwise, enter 0. In this case, the sign of the Z component is changed (UP is plus).

Enter 1 to plot velocities, 0 to plot acceleration, or 2 to plot accelerations.

Enter 1 to use the band pass filter. Otherwise 0.

Enter the four corner frequencies of the trapezoidal band pass filter. In this case, a high cut filter tapering from 6 Hz to 8 Hz is used.

Enter 1, when you use a shorter delta time (dt). Otherwise enter 0. In this case, dt=0.0625 s.

Enter 1, when you change the unit of m to that of cm. Otherwise enters 0.

4. Comparison of Simulation Results with the Observations

We compare simulated and observed strong motions, as shown in examples in Figures 4, 5 (rock sites), 6 and 7 (sediment sites), The observed records downloaded from PEER Strong Motion Database (http://peer.berkeley.edu/smcat/index.html). All the data are high-cut filtered tapering from 1.0 Hz to 1.6 Hz, and all the horizontal data are rotated in the fault normal (N32E) and parallel (N58W) direction (see Figure 1).

Figure 4. Comparison of velocities at the rock sites (SSU and U56)

between the simulations and the observations.

Figure 5. Comparison of accelerations at the rock sites (SSU and U56)

between the simulations and the observations.

Figure 6. Comparison of velocities at the soil sites (NHL, SYM and U03)

between the simulations and the observations.

Figure 7. Comparison of velocities at the soil sites (NHL and SYL)

between the simulations and the observations.